SO SMART APPS LLC

QuadTrack — Hardware Design & Prototype RFQ

Leon Herbert, Founder · leon@sosmartapps.app · 513-601-5092 · Tucson, AZ · July 2026 · Confidential

# Project Overview

QuadTrack is a concealed cellular GPS tracker for Alzheimer’s / dementia patients. It is a self-charging insert that fits a Quad Lock phone-case ring mount — invisible to the patient and impossible to forget like a wearable. Its defining feature: when the patient’s phone battery dies, the tracker keeps reporting and automatically escalates to emergency tracking, alerting all caregivers.

The companion software is complete and live: the Lumina caregiver app (Flutter, iOS/Android) with device dashboard, live maps, tracking-mode control, push alerts, and a law-enforcement missing-person share package, plus a deployed Firebase backend with ping-ingest and device-command endpoints. Only the hardware is being sourced. A working nRF9160-DK is in hand.

# Scope of Work Requested

- Hardware — Electrical design: schematic, PCB layout, BOM.

- RF / antenna — Cellular + GNSS antenna design, tuning, and test within the insert form factor; SAR pre-compliance.

- Power — Qi receiver / energy-harvesting subsystem and LiPo charging & fuel gauge.

- Mechanical — Enclosure/insert design compatible with Quad Lock case ring.

- Prototype build — 5–10 assembled, working EVT units talking to our live backend.

- DFM + certification — DFM package and support through FCC / PTCRB / SAR / UL 2054 / BT SIG / Qi.

- Optional — Zephyr application firmware — quote separately (may be built in-house).

# Technical Requirements

| Area | Requirement |
| --- | --- |
| Core SiP | Nordic nRF9160 (LTE-M / NB-IoT + GNSS). Design MUST follow the Nordic nRF9160 Hardware Integration Guide (nWP-037) exactly to preserve FCC/PTCRB certified-chipset data reuse. |
| Short-range radio | BLE 5.0 link to host phone. Companion Nordic SoC (e.g., nRF52 series) acceptable. |
| Power | LiPo battery + Qi wireless power receiver harvesting from the host phone’s reverse wireless charging. Device reports charging state: qi / reverse / pogo-pin / on-battery. |
| Form factor | Insert that fits a Quad Lock phone-case ring mount; fully concealed from the wearer. Industrial/mechanical design in scope. |
| Antennas / RF | Cellular + GNSS antennas within the insert form factor, adjacent to phone body and Quad Lock mount hardware. Body-worn SAR compliance required. |
| Core behavior | Continues reporting independently when the host phone battery dies and auto-escalates to emergency cadence. Server-driven reporting interval via nextPing response. |
| Cadence | Normal ~30 min; emergency 1–15 min (battery-aware); idle ~4 h. Exact values server-controlled. |
| Telemetry | Each ping: GNSS fix, accuracy, tracker battery %, host phone battery %, charging state, source. |
| Cloud interface | Backend is LIVE (Firebase Cloud Functions): HTTPS ping ingest with X-Device-Key auth + HTTPS command-polling endpoint. JSON schemas provided on request. No cloud work in scope. |
| Firmware | Zephyr / nRF Connect SDK. Base application firmware may be developed in-house — quote firmware as a SEPARATE line item. |
| Certifications | FCC (Part 15 + intentional radiator), PTCRB (chipset data reuse), SAR, UL 2054 (battery), Bluetooth SIG, Qi (WPC). Certification support in scope; lab fees billed separately. |
| Volumes | EVT prototypes: 5–10 units. Provide estimated unit cost at 1k / 5k / 10k units. |

# Quote Requirements

- NRE broken out by phase: architecture, schematic/layout, antenna design + test, mechanical, EVT build, DVT/certification support, and firmware (separate line).

- Timeline per phase with key milestones and dependencies.

- EVT prototype quantity and cost; estimated per-unit cost at 1k / 5k / 10k.

- IP: full ownership of all design outputs (schematics, layout, CAD, firmware, test data) assigned to So Smart Apps LLC. This is mandatory.

- Production path: in-house manufacturing or named CM partners, and what transition to volume production looks like.

# Questions for Your Team

- Prior nRF9160 designs shipped, and experience keeping a design within the Nordic HIG for certification data reuse?

- Experience with Qi receiver / wireless energy-harvesting designs?

- In-house antenna chamber and SAR pre-test capability, or named lab partner?

- Reference customers for a comparable small cellular tracker taken from concept through PTCRB?